# Python DAQ User Guide

Data_Run_bmotion.py performs a data acquisition loop using LeCroy scopes and moves one or more probes through a grid of positions, including irregular grids, using the bapsf_motion library.

It runs two parallel processes:

- Acquire: arm scopes for trigger (SINGLE), record shots after scope is triggerd (STOP), and writes each shot as bin files into “spool” folder.

- Offload: automatically launched in a second console window, copies binary data into the HDF5 file, verifies data integrity, and deletes the binary data.

You only need to run Data_Run_bmotion.py. The offload window opens automatically.

# STEP 0

- Connect scope to lab's internal DAQ network.

- When the Ethernet cable is plugged in, a 192.168.7.XX address should appear under Utilities Setup > Remote. If it does not, try a different Ethernet port on scope.

- DO NOT change any scope IP addresses. This prevents device conflicts on the DAQ network.

- DAQ network IP table at https://docs.google.com/spreadsheets/d/1Vv0nh2Zysy7qkZQiJwoCBddPjNWCaUrVLmoPI7VuG1g/

- Connect the probe drives and zero them.

- Create or choose a base folder to hold the data run configuration files, for example:

E:\Shadow data\XXX\XXX

# Step 1

The base folder needs to include these three files: (can have other files and subdir)

| File | Purpose |
| --- | --- |
| experiment_config.ini | Set scope IPs, probe drive to run, shots per position, hdf5 path, binary save path... |
| bmotion_config.toml | Motor configuration; can be exported or copied from bmotion PC. |
| description.txt | Experiment notes about the run; Optional but recommended. |

See example_experiment_config.ini and example_description.txt under "C:\Users\daq\Desktop\LAPD_DAQ\" for example format. See Configuration Reference section under README.md for all config.ini options

- ALL displayed traces on scope will be recorded whether or not there is a [channels] entry in the ini file. Missing descriptions will return "No description available" in hdf5 file.

- motion_groups are selected by physical drive name (case-insensitive), e.g. Hermes, Hades

  - all runs all probe drives listed in the TOML file).

  - (Optional) Direction decides whether the probe execute its position array forward or backward

  - (Optional) Execution_order: Interleaved moves all probes together (need same position array size); sequential completes all positions of one probe before moving on to the next.

# Step 2

Open Data_Run_bmotion.py and edit line 54 near the top: base_path = r"E:\Shadow data\XX\XX"

- This is the only line that needs edit in Data_Run_bmotion.py, ideally only once for the experiment week.

- experiment_config.ini, bmotion_config.toml, description.txt requires edit for every data run.

- description.txt can be edited during data run, as the Python program updates it before data run finishes.

# Step 3

From the repo directory, with environment active (.venv), run:

python Data_Run_bmotion.py

DAQ PC keeps all of LAPD_DAQ's dependencies inside a folder called .venv (a "virtual environment"). You must activate this environment before running the script; otherwise, Python will not find the parts it needs. Below are two ways to do it. Pick one.

The one rule to remember: before running anything, your command line must show (.venv) at the start of the line. If you don't see (.venv), the environment is not active and the script will fail.

## Method 1 — Using VS Code

VS Code is the program with this blue icon  . It has a built-in command line at the bottom.

- Launch VS Code. Go to the menu File → Open Folder…, navigate to the LAPD_DAQ folder, and click Select Folder. The file list on the left should now show files like Data_Run_bmotion.py.

- From the top menu, choose Terminal → New Terminal. A command-line panel opens at the bottom of the window.

- The line should begin with (.venv), like this: (.venv) C:\Users\...\LAPD_DAQ>

- If (.venv)is missing, run the following command: venv\Scripts\activate.bat

- Run python Data_Run_bmotion.py

- The run begins, and a second window (the offload) opens on its own.

## Method 2 — Using Windows PowerShell

PowerShell is a built-in Windows command line with icon .

- Click the Windows Start button and find Windows PowerShell.

- You need to move into the LAPD_DAQ folder. Type cd (the letters c, d, then a space), then type the full path to the folder, and press Enter.

- Type this exactly and press Enter: .\.venv\Scripts\Activate.ps1

- The start of the line should now show(.venv) PS C:\...\LAPD_DAQ>

- Run the script python Data_Run_bmotion.py

# What happens next

- Once data run starts successfully, you should see some print statements in the terminal window including scope initialization, selected motors, total positions, total shots etc.

- The parallel program that writes binary data into hdf5 will be running until it says data write completed; please close it then.

- A progress bar appears. A rough time estimate will show and a more accurate time estimate will show up once the first line scan completes for 2D scans.